| File Name | Primary Functionality |
| --- | --- |
| config.py | Holds all static parameters (track dimensions, vehicle limits, simulation speed). This prevents "magic numbers" in your code. |
| track.py | Contains the geometry math. It converts the controller's Frenet coordinates ($s$, $e_y$) into Cartesian coordinates for plotting. |
| dynamics.py | Defines the kinematic bicycle model and the state update equations for the vehicles. |
| smpc_controller.py | The "Brain." This will house the CVXPY convex optimization logic from the paper. |
| main_sim.py | The entry point. It imports everything, sets up the initial scenario, and runs the live animation loop. |

1. The Cost Weights: Q and R

In our Convex Optimization problem, the objective function aims to minimize the cost over the prediction horizon. We do this using two matrices: $Q$ (State Cost) and $R$ (Input/Actuation Cost).

Here is exactly what your current config.py weights are telling the car to do:

The $Q$ Matrix (The "Where I want to be" penalties):

- Q_S = 0.0 (Arc Length): We set this to zero because we don't want the car to stop at a specific coordinate on the track. We just want it to keep moving forward. If this were high, the car would slam on the brakes the moment it reached a target distance!

- Q_EY = 50.0 (Lateral Offset): This is very high. It means "stay strictly in the center of your target lane." Any drift left or right is heavily penalized.

- Q_EPSI = 10.0 (Heading Error): A moderate penalty to ensure the car points parallel to the road boundaries.

- Q_V = 20.0 (Speed Error): A strong penalty for deviating from the target speed (V_REF = 15 m/s).

The $R$ Matrix (The "How hard I want to work" penalties):

- R_A = 1.0 (Acceleration Effort): This is very low. We are giving the controller permission to freely use the gas and brake pedals to maintain that 15 m/s target speed.

- R_YAW = 100.0 (Steering Effort): This is the highest penalty of all! It acts as a "comfort" parameter. By making steering mathematically "expensive," the optimizer will avoid jerky, sudden wheel movements, resulting in smooth lane changes and cornering.